1. Connect the DC2100 to the PC through the serial port cable.

2. Get the DC2100 ready to be programmed.

A. With the unit in the off state press and hold T V Y, and

Press F1 to turn the DC2100 on.

B. Answer the questions as follows.

LOAD DEFAULTS        F1-YES

ARE YOU SURE         F1-YES

CLEAR FLASH          F1-YES

ARE YOU SURE         F1-YES

TEST FLASH           F5-NO

C. Now the DC2100 is ready

3. Get the Computer ready.

A. With the file DC2100.HEX and TTLOAD.EXE in the same subdirectory

type the following.

TTLOAD DC2100.HEX

B. The PC will then start the program, and communicate with the DC2100.

There will a series of beeps from the DC2100. This indicates that there

is a connection established.

C. Press the space bar to all of the questions asked, when finished the

DC2100 will display the Main Menu and the PC will return to a DOS prompt

TTLOAD

Load an Intel(tm) Hex file from a personal computer to a DC2100 unit.

Interactively write file to RAM and flash EPROM, then map

intercepts.

Syntax    ttload [/r file.ext][/f file.ext][/bMmn2] [file.hex]

Where 'file.ext' is the name of the Intel� hex file to

load.

Switches  /b   Batch mode.  Performs TTLOAD functions without

interactive response.

/2   Specifies personal computer port COM2: is used for

communications with DC2100 unit.  Default value is

COM1:.  Program supports either COM1: or COM2:.

/h   Program displays a help screen listing these option

switches with a brief description of each.

Examples

COMMAND                     RESULTS

ttload file                 Loads file to RAM, writes RAM to FLASH, and maps

intercepts in interactive mode.

ttload /b file              Loads file to RAM, writes RAM to FLASH, and maps

intercepts in batch mode.

Error Messages

Error Message                      Recommended Action

Checksum error                     Calculated checksum did not match value

returned from DC2100 unit.  Return unit to

default state and try again.

Flash write disabled.              Intercepts have been mapped and must now

be cleared.  Make sure DC2100 unit is in

default state.

File 'file.ext' not found.         Unable to find specified file.

Unable to establish                Ensure proper cable connection from com-

communications with PDS unit.      puter to DC2100 unit.  Make sure DC2100

unit is in default state.1

Write not allowed.                 These errors are returned from the DC2100

unit.  They indicate either an incorrect

Improper EOL or BOL.               hex file format or that the state of the

DC2100 unit is not default.1

ABORT detected or improper

ASCII in line header.

ABORT detected or improper

ASCII in line.

Checksum improper.